SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER ARCHITECTURE

COURSE CODE: CSA1205

COURSE OUTCOME COVERED

CO2: Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems.(BL3)Weightage: 20%

QUESTIONS:5

Code of Conduct:

Total Marks:25

ANNEXURE A

I M.Preethivarthana (192521334) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. 
I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

M.Preethivarthana

1. A program performing signed integer addition using 2’s complement produces incorrect results when adding large values (e.g., +120 and +50 in 8-bit representation).Debug the issue by analyzing the binary computation and identifying whether overflow has occurred. Propose a solution to correctly detect and handle overflow in such cases.

### Problem Statement

A program performs signed integer addition using 2's complement in an 8-bit system. While adding two large positive numbers (+120 and +50), it produces an incorrect negative result. Debug the computation, determine whether overflow has occurred, and propose a method to detect and handle overflow.

## Step 1: Binary Representation

| Decimal | 8-bit Binary |
| --- | --- |
| +120 | 01111000 |
| +50 | 00110010 |

## Step 2: Perform Binary Addition

01111000   (+120)

+ 00110010   (+50)

-------------

10101010

Result (Binary): 10101010

Step 3: Interpret the Result

Since the Most Significant Bit (MSB) = 1, the result is interpreted as a negative number in 2's complement.

Finding its decimal value:

10101010

Invert bits  → 01010101

Add 1        → 01010110 = 86

Therefore,

10101010 = -86

Actual mathematical result:

120 + 50 = 170

But an 8-bit signed integer can only represent values from:

-128 to +127

Since 170 > 127, the value cannot be represented.

## step 4: Debugging Analysis

Expected Result: +170

Obtained Result: -86

Cause of Error: Signed Integer Overflow

Overflow occurs because:

- Two positive numbers were added.

- The result became negative.

- The mathematical result exceeded the maximum 8-bit signed value (+127).

## Step 5: Overflow Detection

Overflow in signed addition occurs when:

- Positive + Positive → Negative

- Negative + Negative → Positive

Another detection method:

Overflow = Carry into MSB XOR Carry out of MSB

If the XOR result is 1, overflow has occurred.

## Step 6: Optimised Solution

To correctly handle overflow:

- Perform the addition.

- Check the sign bits of both operands and the result.

- If overflow is detected:

  - Display an overflow error message.

  - Use a larger data type (16-bit or 32-bit integer).

  - Prevent storing the incorrect value.

### Pseudocode

result = A + B

if ((A > 0 && B > 0 && result < 0) ||

(A < 0 && B < 0 && result > 0))

{

print("Overflow Detected")

}

else

{

print(result)

}

## Answer

- Operands: +120 (01111000) and +50 (00110010)

- Binary Result: 10101010

- Obtained Decimal Result: −86

- Expected Decimal Result: 170

- Overflow Occurred: Yes

- Reason: The result exceeds the 8-bit signed integer range (−128 to +127).

- Optimised Handling: Detect overflow using sign-bit comparison (or carry-in/carry-out XOR) and use a larger integer type or report an overflow error instead of accepting the incorrect result.

2. A processor using a ripple carry adder shows significant delay in executing arithmetic operations in a real-time system. Debug the performance bottleneck by examining how carry propagates through each bit. Suggest an optimized solution using a carry look-ahead adder, and explain how it reduces delay.

## Problem Statement

A processor uses a Ripple Carry Adder (RCA) for arithmetic operations. During execution in a real-time system, arithmetic operations take longer than expected because each carry must propagate through every bit before the final result is produced. Debug the performance bottleneck and propose an optimized solution using a Carry Look-Ahead Adder (CLA).

## Step 1: Analyze the Ripple Carry Adder

Consider adding two 4-bit numbers:

A = 1011

B = 0110

### Binary Addition

Carry: 0 → 1 → 1 → 1

1011

+ 0110

--------

10001

In a Ripple Carry Adder:

- Bit 0 computes first.

- Carry is passed to Bit 1.

- Bit 1 waits for the carry.

- Bit 2 waits again.

- Bit 3 also waits.

Thus, every stage must wait for the previous carry.

## Step 2: Debugging Analysis

### Problem Identified

The processor experiences high propagation delay because:

- Each full adder cannot operate until the previous carry is available.

- Delay increases with the number of bits.

For an n-bit Ripple Carry Adder:

Total Delay ≈ n × Carry Delay

Example:

- 4-bit RCA → 4 carry delays

- 8-bit RCA → 8 carry delays

- 32-bit RCA → 32 carry delays

This makes Ripple Carry Adders unsuitable for real-time applications where fast computation is required.

## Step 3: Performance Bottleneck

Issue: Sequential carry propagation.

Bit0 → Bit1 → Bit2 → Bit3

Each bit must wait for the previous carry.

Therefore,

- Arithmetic operations become slow.

- Processor execution time increases.

- Real-time performance is affected.

## Step 4: Optimised Solution – Carry Look-Ahead Adder (CLA)

Instead of waiting for carries to ripple through each stage, a Carry Look-Ahead Adder computes all carry values simultaneously.

It uses two signals:

### Generate (G)

G = A × B

A carry is generated.

### Propagate (P)

P = A ⊕ B

The carry is propagated.

Using these signals, the carry equations are:

C1 = G0 + P0C0

C2 = G1 + P1G0 + P1P0C0

C3 = G2 + P2G1 + P2P1G0 + P2P1P0C0

Thus, all carries are calculated in parallel rather than one after another.

## Step 5: Optimisation Benefits

| Ripple Carry Adder | Carry Look-Ahead Adder |
| --- | --- |
| Carry propagates one bit at a time | Carries computed simultaneously |
| High propagation delay | Very low delay |
| Slower execution | Faster execution |
| Suitable for small circuits | Suitable for high-speed processors |
| Delay increases linearly with bits | Delay grows much more slowly |

## Step 6: Final Debugging Result

- Problem: Slow arithmetic operations due to sequential carry propagation in the Ripple Carry Adder.

- Cause: Every full adder waits for the carry from the previous stage, creating cumulative delay.

- Optimised Solution: Replace the Ripple Carry Adder with a Carry Look-Ahead Adder (CLA).

Why CLA is Better:

- Computes carries in parallel.

- Greatly reduces propagation delay.

- Improves processor speed.

- Suitable for real-time systems requiring fast arithmetic operations.

## Answer

- Performance Bottleneck: Sequential carry propagation in the Ripple Carry Adder.

- Cause: Each bit waits for the previous carry, increasing delay as the number of bits increases.

- Optimised Solution: Use a Carry Look-Ahead Adder (CLA).

- Advantage: CLA computes carry bits in parallel using Generate (G) and Propagate (P) signals, significantly reducing delay and improving arithmetic performance in real-time systems.

3. A multiplication unit implementing Booth’s algorithm gives incorrect results for certain negative operands. Debug the step-by-step execution of the algorithm, focusing on bit-pair checking, arithmetic shifts, and sign extension. Identify the possible source of error and suggest corrections to ensure accurate signed multiplication.

## Problem Statement

A multiplication unit implementing Booth’s Algorithm produces incorrect results when multiplying certain negative operands. Debug the algorithm by examining the bit-pair checking, arithmetic right shifts, and sign extension. Identify the source of the error and propose corrections to ensure accurate signed multiplication.

## Step 1: Example

Multiply:

Multiplicand (M) = -5

Multiplier (Q)   = 3

Expected Result  = -15

Using 4-bit 2's complement:

| Value | Binary |
| --- | --- |
| -5 | 1011 |
| +3 | 0011 |

Initialize:

A  = 0000

Q  = 0011

Q-1 = 0

## Step 2: Booth's Algorithm Execution

### Initial State

A = 0000

Q = 0011

Q-1 = 0

### Bit-Pair Checking

Check the pair (Q0, Q-1) in every cycle.

| Q0 Q-1 | Operation |
| --- | --- |
| 00 | No operation |
| 01 | A = A + M |
| 10 | A = A − M |
| 11 | No operation |

After the required operation, perform an arithmetic right shift on A, Q, and Q-1.

Repeat this process for the number of bits (4 cycles for a 4-bit multiplier).

## Step 3: Debugging Analysis

The multiplication gives incorrect results because of one or more of the following errors.

### Error 1: Incorrect Bit-Pair Checking

The control unit may incorrectly interpret the pair (Q0, Q-1).

Example:

Q0 Q-1 = 10

Correct action:

A = A − M

If the hardware performs A + M instead, the final product becomes incorrect.

### Error 2: Incorrect Arithmetic Right Shift

An arithmetic right shift must preserve the sign bit.

Correct example:

Before Shift

A = 1010

After Arithmetic Shift

A = 1101

If a logical shift is used instead:

1010 → 0101

The sign bit is lost, producing an incorrect result.

### Error 3: Sign Extension Error

Negative numbers must retain their sign during operations.

Example:

-5 = 1011

Incorrect extension:

001011

Correct sign extension:

111011

Improper sign extension changes the operand value and results in incorrect multiplication.

## Step 4: Possible Source of Error

The incorrect multiplication result may be caused by:

- Wrong interpretation of (Q0, Q-1).

- Logical shift instead of arithmetic right shift.

- Improper sign extension of negative operands.

- Incorrect 2's complement representation of negative numbers.

## Step 5: Optimized Solution

To ensure correct signed multiplication:

- Correctly evaluate every (Q0, Q-1) pair.

- Use arithmetic right shifts to preserve the sign bit.

- Apply proper sign extension for all negative operands.

- Verify the 2's complement representation before multiplication.

- Test the implementation with positive, negative, and mixed-sign operands.

## Final Debugging Result

## Problem: Incorrect multiplication for negative operands.

Cause: Errors in bit-pair checking, arithmetic right shifting, or sign extension.

Optimized Solution: Implement accurate Booth control logic, perform arithmetic right shifts that preserve the sign bit, and correctly sign-extend all negative operands. This ensures reliable signed multiplication for all input combinations.

Answer

- Performance Issue: Incorrect results when multiplying negative numbers.

- Possible Causes:

  - Incorrect (Q0, Q-1) bit-pair interpretation.

  - Logical right shift used instead of arithmetic right shift.

  - Improper sign extension of negative operands.

  - Incorrect 2's complement representation.

- Optimised Solution: Use correct Booth decision logic, preserve the sign bit during arithmetic shifts, ensure proper sign extension, and validate operands before multiplication. These corrections guarantee accurate signed multiplication using Booth’s Algorithm.

4. An embedded system using the restoring division algorithm experiences inefficiency due to repeated restoration steps, leading to increased execution time. Debug the algorithm’s workflow and identify where unnecessary operations occur. Propose an optimized approach using the non-restoring division method, explaining how it improves performance.

## Problem Statement

An embedded system uses the Restoring Division Algorithm for division operations. The system experiences high execution time because the algorithm repeatedly restores the previous value whenever subtraction produces a negative result. Debug the algorithm's workflow, identify the unnecessary operations, and propose an optimized solution using the Non-Restoring Division Algorithm.

## Step 1: Analyze the Restoring Division Algorithm

Example:

Dividend = 13

Divisor = 3

Expected Quotient = 4

Expected Remainder = 1

Initial Registers:

A = 0000      (Accumulator)

Q = 1101      (Dividend = 13)

M = 0011      (Divisor = 3)

### Workflow

For each bit:

- Left shift (A, Q).

- Subtract divisor from accumulator.

- Check the sign of A.

- If A is negative:

  - Restore by adding the divisor back.

  - Set quotient bit = 0.

- Otherwise:

  - Keep the result.

  - Set quotient bit = 1.

Repeat until all bits are processed.

## Step 2: Debugging Analysis

### Problem Identified

Whenever subtraction gives a negative value, the algorithm performs an extra restoration step.

Example:

A = A - M

If A < 0

A = A + M   ← Restoration

This additional addition is repeated many times during the division process.

## Step 3: Performance Bottleneck

The unnecessary operation is:

Subtract → Negative Result → Restore → Continue

This means:

- Extra addition operations are executed.

- More clock cycles are required.

- Execution time increases.

- Processor efficiency decreases.

## Step 4: Optimized Solution – Non-Restoring Division

The Non-Restoring Division Algorithm eliminates the repeated restoration step.

Instead of immediately restoring the accumulator:

- If the previous remainder is positive, subtract the divisor.

- If the previous remainder is negative, add the divisor in the next cycle.

Thus, restoration is postponed and handled naturally in the next iteration.

Workflow:

Positive Remainder

↓

Subtract Divisor

Negative Remainder

↓

Add Divisor (Next Cycle)

No immediate restoration is required.

## Step 5: Advantages of Non-Restoring Division

| Restoring Division | Non-Restoring Division |
| --- | --- |
| Restores after every negative result | No immediate restoration |
| More arithmetic operations | Fewer arithmetic operations |
| Higher execution time | Faster execution |
| More clock cycles | Reduced clock cycles |
| Lower processor efficiency | Higher processor efficiency |

## Step 6: Optimization Benefits

Using the Non-Restoring Division Algorithm:

- Eliminates unnecessary restoration operations.

- Reduces the number of additions and subtractions.

- Decreases execution time.

- Improves processor speed.

- Increases efficiency in embedded and real-time systems.

## Final Debugging Result

Problem: Slow execution due to repeated restoration after negative subtraction.

Cause: The restoring algorithm performs unnecessary addition operations to restore the accumulator whenever the subtraction result is negative.

Optimized Solution: Replace the Restoring Division Algorithm with the Non-Restoring Division Algorithm, which avoids immediate restoration by correcting the remainder in the next iteration.

Answer

- Performance Bottleneck: Repeated restoration after every negative subtraction.

- Cause: Extra addition operations increase clock cycles and execution time.

- Optimized Solution: Use the Non-Restoring Division Algorithm.

- Advantage: It removes unnecessary restoration steps, performs fewer arithmetic operations, reduces execution time, and improves overall processor performance, making it more suitable for embedded and real-time systems.

5. A scientific application using IEEE 754 single precision floating-point representation produces inaccurate results when adding very small and very large numbers. Debug the issue by analyzing exponent alignment, normalization, and rounding errors. Suggest optimization techniques, such as using double precision or algorithmic adjustments, to improve accuracy.

## Problem Statement

A scientific application uses IEEE 754 Single Precision (32-bit) floating-point representation. While adding a very large number and a very small number, the program produces inaccurate results. Debug the computation by analyzing exponent alignment, normalization, and rounding errors, and suggest optimization techniques to improve accuracy.

## Step 1: Example

Consider adding:

Large Number = 1.0 × 10^8

Small Number = 1.0

Expected Result:

100000001

However, the program outputs:

100000000

The small number is lost during the addition.

## Step 2: IEEE 754 Single Precision Representation

A 32-bit floating-point number consists of:

| Field | Bits |
| --- | --- |
| Sign | 1 |
| Exponent | 8 |
| Mantissa (Fraction) | 23 |

Before addition, both numbers must have the same exponent.

## Step 3: Debugging Analysis

### 1. Exponent Alignment

Before adding, the floating-point hardware aligns the exponents.

Large Number : 1.000000 × 2^26

Small Number : 1.000000 × 2^0

To match the larger exponent, the mantissa of the smaller number is shifted many times to the right.

1.000000 → 0.000000000000...

After repeated shifting, the significant bits of the small number are discarded.

### 2. Normalization

After addition, the result is normalized so that only one non-zero digit remains before the binary point.

Example:

10.101100 × 2^5

becomes

1.0101100 × 2^6

Normalization may shift the mantissa again, causing further loss of precision.

### 3. Rounding Error

Single precision stores only 23 bits of the mantissa.

Extra bits are rounded off.

Example:

Actual Value

1.101011110011001101010111...

Stored Value

1.101011110011001101011

The discarded bits introduce rounding errors.

## Step 4: Source of Error

The inaccurate result is caused by:

- Large exponent difference between the operands.

- Loss of significant digits during exponent alignment.

- Normalization shifting the mantissa.

- Rounding because only 23 mantissa bits are available.

This phenomenon is known as loss of precision.

## Step 5: Optimized Solution

### 1. Use Double Precision (64-bit)

Double precision provides:

- 11 exponent bits

- 52 mantissa bits

Benefits:

- Higher precision.

- Smaller rounding errors.

- Better accuracy for scientific calculations.

### 2. Algorithmic Adjustments

Improve numerical accuracy by:

- Adding numbers from smallest to largest instead of largest to smallest.

- Grouping numbers with similar magnitudes before addition.

- Using compensated summation methods (such as Kahan Summation) to reduce rounding errors.

- Avoiding unnecessary repeated floating-point operations.

## Step 6: Optimization Benefits

| Single Precision | Double Precision |
| --- | --- |
| 32 bits | 64 bits |
| 23-bit mantissa | 52-bit mantissa |
| Lower precision | Higher precision |
| Larger rounding errors | Smaller rounding errors |
| Suitable for general applications | Suitable for scientific and engineering applications |

## Final Debugging Result

Problem: Inaccurate addition when combining very small and very large floating-point numbers.

Cause: Exponent alignment shifts the smaller operand, causing loss of significant digits. Normalization and rounding further reduce precision.

Optimized Solution: Use IEEE 754 Double Precision (64-bit) for higher accuracy and apply algorithmic techniques such as adding smaller numbers first or using compensated summation to minimize rounding errors.

Answer

- Issue: Loss of precision when adding very small and very large floating-point numbers.

- Cause: Exponent alignment, normalization, and rounding errors in IEEE 754 single precision.

- Optimized Solution: Use double precision (64-bit) to increase mantissa size and reduce rounding errors. Improve numerical accuracy by adding numbers in order of increasing magnitude and using compensated summation algorithms.