SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER ARCHITECTURE

COURSE CODE: CSA1205

COURSE OUTCOME COVERED

CO2: Apply integer and floating-point arithmetic algorithms including Booth's  multiplication, restoring/non-restoring division, and IEEE 754 standard  operations to solve fixed-point and floating-point computational problems.  (BL3) weightage:45%

QUESTIONS:5

Code of Conduct:

Total Marks:25 APPLICATION BASED QUESTIONS

ANNEXURE A

I M.Preethivarthana (192521334) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. 
I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Preethivarthana.M

- A processor performs arithmetic operations on signed 8-bit integers using 2’s complement representation. Consider two numbers: +45 and −23. Analyze in detail how the system performs both addition and subtraction, including binary conversion, alignment of sign bits, and intermediate steps. Determine whether overflow occurs, and explain how the processor detects overflow and interprets the final result.

### Given

- Number 1 = +45

- Number 2 = −23

The processor uses 8-bit 2's complement representation.

## Step 1: Convert Decimal to Binary

### +45

45 = 00101101

+45 = 00101101

### −23

First convert +23 into binary.

23 = 00010111

Take 2's complement.

00010111

Invert

11101000

+1

---------

11101001

Therefore,

−23 = 11101001

# (A) Addition : +45 + (−23)

00101101

+ 11101001

------------

1 00010110

Ignore carry out.

Result

00010110

Decimal = 22

### Overflow Check

Overflow occurs only when

- Two positive numbers produce negative result

- Two negative numbers produce positive result

Here,

Positive + Negative = Positive

No Overflow

# (B) Subtraction : +45 − (−23)

Subtracting a negative means adding its positive value.

Need +23.

+23 = 00010111

Now,

00101101

+00010111

-----------

01000100

Result

01000100

Decimal = 68

No overflow because answer lies within

−128 to +127

## Overflow Detection

Processor checks carry into and carry out of the sign bit.

If

Carry into MSB ≠ Carry out of MSB

Overflow occurred.

Otherwise,

No overflow.

Here,

Carry into sign bit = Carry out

Hence,

No Overflow

## Final Results

| Operation | Binary Result | Decimal | Overflow |
| --- | --- | --- | --- |
| 45 + (−23) | 00010110 | 22 | No |
| 45 − (−23) | 01000100 | 68 | No |

2. A high-performance computing system replaces a ripple carry adder with a 4-bit carry look ahead adder (CLA) to reduce computation delay. Given two binary inputs, analyze how  generate and propagate signals are formed and how carries are computed in advance. Compare the time delay of CLA with ripple carry addition and justify, with reasoning, why  CLA improves speed in modern processors.

Given two 4-bit numbers

A = 1011

B = 0110

## Step 1: Generate (G) and Propagate (P)

Formulas

Generate:

Gi = Ai × Bi

Propagate:

Pi = Ai ⊕ Bi

| Bit | A | B | Gi | Pi |
| --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 |
| 2 | 0 | 1 | 0 | 1 |
| 3 | 1 | 0 | 0 | 1 |

## Step 2: Carry Equations

C1 = G0 + P0C0

C2 = G1 + P1G0 + P1P0C0

C3 = G2 + P2G1 + P2P1G0 + P2P1P0C0

C4 = G3 + P3G2 + P3P2G1 + P3P2P1G0 + P3P2P1P0C0

Assume

C0 = 0

Carries are calculated simultaneously instead of waiting.

## Ripple Carry Delay

Each full adder waits for previous carry.

Delay

FA1 → FA2 → FA3 → FA4

If one adder takes

t

Then

Ripple Delay = 4t

## CLA Delay

Carries are computed together.

Approximate delay

Gate delay ≈ 2t

## Comparison

| Ripple Carry Adder | Carry Look Ahead Adder |
| --- | --- |
| Carry generated sequentially | Carry generated simultaneously |
| Slower | Faster |
| Simple design | More hardware |
| Delay increases with bits | Very small delay |

3. In a signed multiplication operation, a processor uses Booth’s algorithm to multiply two  numbers, −6 and +5. Analyze the algorithm step-by-step, including the role of the  accumulator, multiplier, and Booth bit, along with arithmetic shifts. Explain how the  algorithm efficiently handles signed numbers and reduces the number of addition/subtraction  operations compared to traditional multiplication.

### Given

Multiplicand (M) = −6

Multiplier (Q) = +5

Using 4 bits

−6 = 1010

+5 = 0101

Initialize

A = 0000

Q = 0101

Q−1 = 0

## Booth Rules

| Q0Q−1 | Operation |
| --- | --- |
| 00 | No operation |
| 11 | No operation |
| 01 | A = A + M |
| 10 | A = A − M |

After every operation

Perform Arithmetic Right Shift.

## Iteration 1

Q0Q−1 = 10

A = A − M

Shift

## Iteration 2

Q0Q−1 = 01

A = A + M

Shift

## Iteration 3

Q0Q−1 = 10

Subtract

Shift

## Iteration 4

Q0Q−1 = 01

Add

Shift

Final Product

11100010

Decimal

−30

Correct because

−6 × 5 = −30

## Components

| Register | Purpose |
| --- | --- |
| A | Accumulator |
| Q | Multiplier |
| M | Multiplicand |
| Q−1 | Previous multiplier bit |

4. A processor is required to perform division of two binary numbers (e.g., 13 ÷ 3) using both  restoring and non-restoring division algorithms. Analyze both methods step-by-step, showing  intermediate remainders and quotient formation. Compare the two approaches in terms of  number of steps, complexity, and efficiency, and explain why non-restoring division is often  preferred in modern systems.

Convert to binary

13 = 1101

3 = 0011

# (A) Restoring Division

Initial

A = 0000

Q = 1101

M = 0011

### Step 1

Shift AQ

Subtract divisor

A = A − M

Negative

Restore

A = A + M

Q0 = 0

### Step 2

Repeat

Positive

Q0 = 1

Continue for four iterations.

Final

Quotient = 0100

Remainder = 0001

Decimal

13 ÷ 3

Quotient = 4

Remainder = 1

# (B) Non-Restoring Division

Instead of restoring every time,

If remainder negative

Next operation

Add divisor

If remainder positive

Subtract divisor

After four iterations

Quotient = 0100

Remainder = 0001

## Comparison

| Feature | Restoring | Non-Restoring |
| --- | --- | --- |
| Restore step | Yes | No |
| Operations | More | Less |
| Execution speed | Slower | Faster |
| Hardware | Simple | Slightly complex |
| Efficiency | Lower | Higher |

5. A scientific application requires precise handling of real numbers using the IEEE 754  standard for floating-point representation. Given a decimal number (e.g., −13.25), analyze  how it is represented in both single precision and double precision formats, including sign,  exponent, and mantissa fields. Further, explain how floating-point addition is performed  between two such numbers, highlighting issues like normalization, rounding, and precision  loss.

Given

−13.25

## Step 1: Convert to Binary

Integer

13 = 1101

Fraction

0.25 = 0.01

Therefore

13.25 = 1101.01₂

Normalize

1.10101 × 2³

# Single Precision (32-bit)

### Sign Bit

Negative

S = 1

### Exponent

Bias

127

Actual exponent

3

Stored exponent

127 + 3 = 130

Binary

10000010

### Mantissa

Remove leading 1

10101000000000000000000

### Final Representation

| Sign | Exponent | Mantissa |
| --- | --- | --- |
| 1 | 10000010 | 10101000000000000000000 |

# Double Precision (64-bit)

Bias

1023

Exponent

1026

Binary

10000000010

Mantissa

1010100000000000000000000000000000000000000000000000

## Floating Point Addition

Suppose

13.25 + 5.5

### Step 1

Convert both to IEEE format.

### Step 2

Align exponents.

Smaller exponent is shifted right.

### Step 3

Add mantissas.

### Step 4

Normalize result.

### Step 5

Round according to IEEE rules.

### Step 6

Store sign, exponent and mantissa.

## Precision Issues

### Normalization

Keeps leading digit as

1.xxxxx

### Rounding

IEEE uses

Round to nearest even

### Precision Loss

Occurs because

- Limited mantissa bits

- Some decimal numbers cannot be represented exactly

- Small rounding errors accumulate during calculations

## IEEE 754 Comparison

| Feature | Single Precision | Double Precision |
| --- | --- | --- |
| Total bits | 32 | 64 |
| Sign | 1 | 1 |
| Exponent | 8 | 11 |
| Mantissa | 23 | 52 |
| Bias | 127 | 1023 |
| Precision | ≈7 decimal digits | ≈15–16 decimal digits |